产品单页介绍 — F1205

文档编号：	SPI-F1205-REV04-2021

产品型号：	F1205

产品名称：	程控直流高斯/特斯拉计

关键词：	30000G/1G DC，精密、高速低噪声

RTN、温度补偿、线性校正、宽温范围

程控、内部存储器

保修服务：	1年有限保修服务（不包含霍尔探头）

校准服务：	1次免费校准

计量服务：	非强制计量产品、付费开具计量证书

使用标准：	GB-1002、EIA-232-E、USB 1.1 Standard、IEC 825-1

产品特点：	 直流测量		 测量范围 3.0000T（30000G）DC

 显示分辨率4 3/4位、1G@30000G

 准确度：（0.3%读数+3G）@ 25C20C/0—5000G

 读数速率：40读数/s

 霍尔探头		 专有RTN零点稳定和降噪技术

 通用黄铜/玻璃纤维封装免校准数字化横向霍尔探头

 通用黄铜封装免校准数字化轴向霍尔探头

 自动温度补偿、多点线性校正

 温度系数	(0.005%读数+0.0005G)/C @ 25C20C

(0.01%读数+0.001G)/C @ 25C40C

 探头自动校零功能

 自动控制		 标准配备RS-232C接口和易用的接口特性

 标准配备内部USB—RS-232C转换接口

 外部触发同步功能、可选触发延迟时间、8192读数触发存储器

 模拟输出功能、可设置为浮动或接地测量

 交互能力		 白色背光FSTN LCD显示器、必备的LED状态指示器

 与显示器和指示器配合的34键交互型功能键盘操作

 数学计算		 最大（最小）值保持和读数滤波功能

 G、kG、T、mT、kA/m、MA/m单位变换

 标准规格		 提供用户手册和设备驱动程序

 标准半宽2U台式仪器，交流220V/50Hz供电

 外形尺寸WHD=21788317mm

标准配置：	 仪器		F1205				程控高斯/特斯拉计					1台

 探头		F12011				数字化玻璃纤维封装横向霍尔探头		1只

 附件		CAB-AC250-052 		3芯交流250V/5A GB1002电源线2m	1条

CAB-232-DCE-3		RS-232C标准直通电缆3m				1条

CAB-USB-AB-2		USB标准A-B电缆1.8m				1条

FUSE5G-25005		250V/0.5A陶瓷保险管，520mm		2只

 光盘		CD-F1205	 		产品信息光盘						1片

内含：				MAN-F1205		用户手册电子文档

DRV-USB232		USB接口驱动程序

可选配置：	探头		F12002				数字化黄铜封装轴向霍尔探头

SINGLE PAGE INTRODUCTION - F1205

Document Number：	SPI-F1205-REV04-2021

Model:	F1205

Name:	Programmable DC Gauss/Tesla Meter

Key words:	30000G/1G DC, precision, high speed low noise

RTN, temperature compensation, linear compensation, wide ambient temperature

programmable, internal memory

Warranty:		Limited warranty for a period of 1 year from delivery（except hall probes）

Calibration:	Free for the first time calibration

Measuration: 	Without force measuration product, payment for Certification of China

Standard:		Follows GB-1002, EIA-232-E, USB 1.1 Standard and IEC 825-1

Specification:	 DC Measurement	 Range of 3.0000T (30000G)

 Display resolution of 4 3/4digit, 1Gs@32000G

 Accuracy of (0.3% READ+3G) @ 25C20C /0-5000G

 40 reading/s

 Probe				 RTN technology for high zero stability and noise rejection

 Brass or fiberglass sealed digital transverse hall probes

 Brass sealed digital axial hall probes

 Temperature and linearity compensation

 TR of 	(0.005%READ+0.0005G)/ C @ 25C20C

(0.01%READ+0.001G)/ C @ 25C40C

 Probe auto zero function

 Automation		 RS-232C interface as standard configuration

 Internal USB-RS232 converter

 External synchronous triggering with optional delay time

 8192 reading trigger memory

 Analog output interface with floating or ground function

 Human Interface	 FSTN LCD display with white back lighting, LED indicators

 34 functional keyboard operation

 Math Functions		 Max/Min hold and display filtering

 G, kG, T, mT, kA/m, MA/m unit conversion

 Standard Spec.		 User’s manual and device drivers

 Standard half rack 2U instrument, AC 220V/50Hz supply

 Dimension of WHD=21788317 mm

Standard Configuration:

 Instruments		F1205 				Programmable Gauss/Tesla Meter	1

 Probes			F12011 				Fiberglass sealed transverse probe	1

 Accessories		CAB-AC250-052		GB1002 AC 250V/5A 3-conductor	1

power cord of 2m long

CAB-232-DCE-3		Std. RS-232C straight 	through		1

cable of 3m long

CAB-USB-AB-2		Std. USB A-B cable of 1.8m long	1

FUSE5G-25005		250V/0.5A ceramic fuse, 520mm	2

 CDs				CD-F1205			Product information CD			1

which contains:		MAN-F1205 user’s manual e-documents

DRV-USB232 interface driver

Options:		 Probes			F12002 				Brass sealed axial probe