产品单页介绍 — F2130

文档编号：	SPI-F2130-REV01-2020

产品型号：	F2130

产品名称：	程控功率电流源

关键词：	10A/40V/400W、真双极性、四象限、强电感

高精度、高稳定度、高分辨率、高速响应、电流渐变

程控、内部存储器、时间常数可调

保修服务：	1年有限保修服务

校准服务：	1次免费校准

计量服务：	非强制计量产品、付费开具计量证书

使用标准：	GB-1002、EIA-232-E、USB 1.1 Standard、IEC 825-1

产品特点：	 输出电流		 输出范围：-10A—+10 A DC，真双极性

 步进分辨率0.01mA（1/1000000 F.S.）

 准确度：（0.02%设置值+0.5mA）

 线性度：优于0.005%

 温度系数：优于10ppm/C

 短期稳定度：50ppm/h

 噪声/纹波：2mA RMS典型值

 响应速度：<100s（电阻负载、-10A至10A、IME模式）

 渐变速率：0.01A/s—10A/s（RAMP模式）

 直流输出阻抗：>100M

 浮动输出，四象限

 输出电压		 有效输出范围：60V DC

 开路输出电压：>65V DC

 输出功率		 最大有效输出功率：600W

 自动控制		 标准配备RS-232C接口和易用的接口特性

 标准配备内部USB—RS-232C转换接口

  3组非易失性存储器、每组存储深度1024个设置值

  键盘触发、接口触发和外部同步触发3种触发输入方式

 包含极头退磁扫描在内的4种模式电流自动扫描功能

 同步触发输出功能

 负载保护		 适应强电感性负载

 可配置的9种时间常数为电感性负载提供最佳响应曲线

 输出端保护防止上电冲击和静电损伤

 输出端状态可控，支持开机更换负载

 交互能力		 白色背光FSTN LCD显示器

 必备的LED状态指示器

 与显示器和指示器配合的35键交互型功能键盘操作

 标准规格		 提供用户手册和设备驱动程序

 标准全宽3U台式仪器，交流220V/50Hz供电

 散热方式：强制风冷

 外形尺寸WHD=434132610mm

标准配置：	 仪器		F2130				程控功率电流源						1台

 附件		CAB-AC250-102 		3芯交流250V/10A GB1002电源线2m	1条

CAB-232-DCE-3		RS-232C标准直通电缆3m				1条

CAB-USB-AB-2		USB标准A-B电缆1.8m				1条

FUSE5G-25040		250V/4A陶瓷保险管，520mm			2只

 光盘		CD-F2130	 		产品信息光盘						1片

内含：				MAN-F2130		用户手册电子文档

DRV-USB232		USB接口驱动程序

Single Page Introduction – F2130

Document Number：	SPI-F2130-REV01-2020

Model:	F2130

Name:	Programmable Power Current Source

Key words:	10A/40V/400W, true bipolar, four-quardrant, strong inductance load

high precision, high stability, high resolution, fast response, current ramp,

programmable, internal memory, configurable time constant

Warranty:		Limited warranty for a period of 1 year from delivery

Calibration:	Free for the first time calibration

Measuration: 	Without force measuration product, payment for Certification of China

Standard:		Follows GB-1002, EIA-232-E, USB 1.1 Standard and IEC 825-1

Specification:	 Output Current		 Range of –10A to +10A DC, true bipolar

 0.01mA step resolution, (1/1000, 000 F.S.)

 Accuracy of  (0.02% settings+0.5mA)

 Linearity of better than 0.005%

 Temperature coefficient of better than 10ppm/C

 Short term stability of better than 20ppm/h

 Noise and ripple of 2mA RMS typical

 100s response, resistive load, –2.5A to 2.5A (IME mode)

 Ramp rate from 0.01A/s to 10A/s (RAMP mode)

 DC output resistance >100M

 Floating output at four-quardrant

 Output Voltage		 Efficient range of 60V DC

 Open circuit output voltage of no less than 65V DC

 Output Power		 Efficient power output of 600W max

 Automation		 RS-232C interface as standard configuration

 Internal USB-RS232 converter

 3 groups of nonvolatile 1024 depth setting memory

 Trigger input of key, interface or external

 Current sweep with 4 modes, including pole degauss

 Synchronous triggering output with optional delay time

 Load Protection	 Capability of driving strong inductance load

 9 selectable time constant for best inductive load response

 Output protection to avoid power-on or static shocks

 Controllable output status allows power-on load connections

 Human Interface	 FSTN LCD display with white back lighting and LED indicators

 35 functional keyboard operation

 Standard Spec.		 User’s manual and device drivers

 Standard full rack 3U instrument, AC 220V/50Hz supply

 Forced air cooling

 Dimension of WHD=434132610 mm

Standard Configuration:

 Instruments		F2130				Programmable Power CS			1

 Accessories		CAB-AC250-102		GB1002 AC 250V/10A 3-conductor	1

power cord of 2m long

CAB-232-DCE-3		Std. RS-232C straight 	through		1

cable of 3m long

CAB-USB-AB-2		Std. USB A-B cable of 1.8m long	1

FUSE5C-25040		250V/4A ceramic fuse, 520mm	2

 CDs				CD-F2130			Product information CD			1

which contains:		MAN-F2130 user’s manual e-documents

DRV-USB232 interface driver